# Tiny Sorter (ESP32-C3 Edition)

Open the instructions page: https://experiments.withgoogle.com/tiny-sorter/view/

## 1) Assemble your sorter (hardware).

When the page shows “Connect wires to the Arduino”, do this instead for the ESP32:
(Use positional servos, not continuous ones)

- Brown → G pin

- Red → 5V pin

- Orange → Pin 4

## 2) Skip their “Connect it to your browser” instructions

We’ll use MicroPython on the ESP32-C3 instead.

## 3) Put the code on the ESP32-C3 using Thonny

- Plug the ESP32-C3 into the laptop with a USB cable

- On the laptop, open Thonny.

- Bottom-right, click the Python interpreter label.

  - If MicroPython (ESP32) is listed, select it.

  - If not: Configure Interpreter… → choose MicroPython (ESP32), install/update if needed.

- Copy files to the device:

  - Open → This Computer → Desktop → Digital Maker Camp → Python

  - Select simple_esp.py, 
then File → Save copy… → MicroPython device → save as simple_esp.py

  - Select sorter.py
then Save copy… → MicroPython device → save as sorter.py

  - Select main.py 
then Save copy… → MicroPython device → save as main.py

- Click the green Run button.
In the Shell you should see it connect to Wi-Fi and print an address like 192.168.8.123. Note this number.

## 4) Open the sorter webpage and point it at your ESP32-C3

- On the desktop, double click sorter.cmd  (Windows) sorter.sh (Linux)

- Click Execute, don’t sign into Chromium (if prompted), and Allow Camera

- Top-left where it shows http://192.168.8.198, replace it with the number you noted (e.g. http://192.168.8.123).

## 5) Go back to the original page

Return to https://experiments.withgoogle.com/tiny-sorter/view/ and continue at Step 3: Train your model.