# Scouts MicroPython Mini-Course

Using the Thonny app and an ESP32-C3 microcontroller device.

## 0) What is Coding?

Coding (or programming) means writing instructions for a computer to follow. You’re the boss — you tell it what to do, and it does exactly what you say (even if it’s not what you meant!).

Think of it like giving instructions to a robot:

1. Pick up a slice of bread
2. Spread butter
3. Add jam
4. Put on another slice

That’s what coding is like — step-by-step instructions. When you write a program, the computer runs through each line in order.

We’ll use the programming language Python, which is popular, easy to read, and used all over the world.

## 1) Getting Set Up

- Install Thonny: https://thonny.org

- Install MicroPython on your ESP32-C3:

  - Plug in the board with a USB-C cable.

  - In Thonny, go to Tools → Options → Interpreter.

  - Choose MicroPython (ESP32), then click Install or update MicroPython.

  - Select ESP32-C3 and follow the steps.

- Select the correct port (bottom-right corner of Thonny).

- You’ll see two panels: This computer and MicroPython device.

Once you’re set up, we’ll write our very first program.

## 2) First Steps in Python — “Hello, world!”

Before using any hardware, let’s write a simple program that just prints something.

### Code:

print("Hello, world!")

Click Run in Thonny. You should see Hello, world! printed in the Shell.

### How it works

- print() tells the computer to display text.

- Text in quotes (" ") is called a string.

- Python reads code top to bottom, one line at a time.

### Coding tips

- Case matters: Print and print are not the same.

- Indentation (spaces) shows which lines belong together (we’ll use it soon).

- You can add comments with # to explain your code.

# This is a comment, it doesn’t change what the code does
print("Hello, Scouts!")

## 3) Variables and Loops — Flashing the LED

Now let’s make the onboard LED blink. You’ll learn about variables, loops, and functions.

### Code: blink_led.py

from machine import Pin
import time

LED_PIN = 8  
led = Pin(LED_PIN, Pin.OUT)

while True:
    led.on()
    time.sleep(0.5)
    led.off()
    time.sleep(0.5)

### New ideas

- A variable (like LED_PIN) stores a value you can use later.

- A loop (while True:) repeats the code inside it forever.

- time.sleep(0.5) pauses for half a second. Change the number to make it slow or fast

## 4) Buttons and Decisions — Button → LED

Next, we’ll make the LED light up only when a button is pressed. This introduces branches (if/else decisions).

### Code: button_led.py

from machine import Pin
import time

LED_PIN = 8      
BUTTON_PIN = 9   

led = Pin(LED_PIN, Pin.OUT)
button = Pin(BUTTON_PIN, Pin.IN, Pin.PULL_UP)

print("Press the button to turn the LED on.")

while True:
    if button.value() == 0:  # pressed
        led.on()
    else:                    # released
        led.off()
    time.sleep_ms(10)

### New ideas

- If / Else: the computer decides what to do based on a condition.

- True / False: buttons and sensors often return True/False or 1/0 values.

- Indentation: lines under if or else must be indented equally.

## 5) Using Functions — Making Reusable Code

Sometimes you want to repeat a set of steps. That’s where functions help.

### Code: blink_function.py

from machine import Pin
import time

led = Pin(8, Pin.OUT)

def blink(times, delay):
    for i in range(times):
        led.on()
        time.sleep(delay)
        led.off()
        time.sleep(delay)

print("Blinking 3 times...")
blink(3, 0.3)

### New ideas

- def blink(times, delay): defines a function — a reusable bit of code.

- for i in range(times): repeats a certain number of times.

- The : starts a code block, and indentation shows which lines belong to it.

Challenge: Write code that flashes the LED quickly four times, then twice slowly.

## 6) Faces on the Display — Changing with Button Presses

Let’s add a simple graphics activity before the weather app. We’ll draw faces on the small display and change them using button clicks. One click, a double click or a holding down for a long click will display different images.

### New ideas right before we use them

- Multiline strings for ASCII art (the face images).

- Events/Callbacks: we won’t keep checking the button; instead, we’ll react when it’s clicked.

- Global variables: when a function needs to change a variable defined outside it, we declare it global.

### Code: happy.py

from simple_esp import SmallDisplay, Input
import time
from machine import Pin

display = SmallDisplay()
inp = Input(9)
led = Pin(8, Pin.OUT)

smile = """
  ####
 #    # 
# #  # #
#      #
# #  # #
#  ##  #
 #    #
  ####  
"""

sad = """
  ####
 #    # 
# #  # #
#      #
#  ##  #
# #  # #
 #    #
  ####  
"""

neutral = """
  ####
 #    # 
# #  # #
#      #
# #### #
#      #
 #    #
  ####  
"""

blank = """
  ####
 #    # 
# #  # #
#      #
#      #
#      #
 #    #
  ####  
"""

smile_image = display.create_image(smile)
sad_image = display.create_image(sad)
neutral_image = display.create_image(neutral)
blank_image = display.create_image(blank)

happy = False

def show_neutral():
    global happy
    display.image(neutral_image)
    display.show()
    happy = False

def show_blank():
    global happy
    display.image(blank_image)
    display.show()
    happy = False

def toggle_image():
    global happy
    happy = not happy
    if happy:
        display.image(smile_image)
        led.value(0)
    else:
        display.image(sad_image)
        led.value(1)
    display.show()

# Wire up button events
inp.on_click = toggle_image
inp.on_long_click = show_neutral
inp.on_double_click = show_blank

# Start state
show_neutral()
led.value(0)

# Idle loop (events do the work)
while True:
    time.sleep(0.1)

## 7) Weather Program — Getting Data from the Internet

Now that you’ve practised functions and events, let’s connect to Wi‑Fi and read the temperature.

### New ideas right before we use them

- Modules: importing ready‑made tools from a helper library.

- Dictionaries: reading data['current']['temperature_2m'] from JSON.

- Error handling mindset: check for connection before fetching.

This URL will display the weather, however it does so in a way that is easy to use in code. It is returned in a format called JSON, a popular format for transferring information/data.

https://api.open-meteo.com/v1/forecast?latitude=52.24&longitude=0.71&current=temperature

Returns:

{

"latitude": 52.239998,

"longitude": 0.7199998,

"generationtime_ms": 0.0386238098144531,

"utc_offset_seconds": 0,

"timezone": "GMT",

"timezone_abbreviation": "GMT",

"elevation": 41,

"current_units": {

"time": "iso8601",

"interval": "seconds",

"temperature": "°C"

},

"current": {

"time": "2025-11-13T16:15",

"interval": 900,

"temperature": 14.1

}

}

### Code: weather.py

from simple_esp import connect_wifi
import urequests as requests

SSID = "scouts"
PASSWORD = "greatbarton"
LAT = 52.24
LON = 0.71  # Great Barton area
URL = f"https://api.openmeteo.com/v1/forecast?latitude={LAT}&longitude={LON}&current=temperature"

wlan = connect_wifi(SSID, PASSWORD)
if not wlan or not wlan.isconnected():
    raise RuntimeError("Wi‑Fi failed — check SSID/PASSWORD")

res = requests.get(URL)
data = res.json()
res.close()

current = data.get("current", {})
temp = current.get("temperature")
print("Temperature:", temp, "°C")

Try this: Print a friendly sentence like It is 12°C — chilly! and change the message based on the temperature.

## 8) Message Program — Two‑Way Messaging (menu + Bluetooth)

Make a simple two‑way messenger. Short‑press cycles through phrases. Long‑press sends. Incoming messages pop up on screen.

### New ideas right before we use them

- Lists: store multiple phrases and cycle through them.

- Slicing & windows: display up to 5 items, with a highlighted cursor.

- Callbacks: functions that run when buttons are pressed or messages arrive.

### Code: message.py

from simple_esp import Input, SmallDisplay, Bluetooth
from machine import Pin
import time

# MENU: Write your phrases here 👇
PHRASES = [
    "Hello",
    "Yes",
    "No",
    "Bye"
]

current = 0
bus = Bluetooth()   
display = SmallDisplay()
led = Pin(8, Pin.OUT)

# DRAW: show up to 5 items, highlight the current one
def draw_menu():
    global current
    start = max(0, min(current - 2, len(PHRASES) - 5))
    view = PHRASES[start:start+5]
    highlight = min(current - start, 4)
    display.display_lines(view, highlight=highlight)

# BUTTON: short press = next item
def on_click():
    global current
    current = (current + 1) % len(PHRASES)
    draw_menu()

# BUTTON: long press = send message
def on_long():
    idx = current
    display.notify("Sending...", ms=300)
    bus.send_index(idx)     # send to other devices
    led.value(1)
    draw_menu()
    time.sleep_ms(500)
    led.value(0)

# RECEIVE: when another device sends a phrase index
def on_receive(idx):
    if 0 <= idx < len(PHRASES):
        msg = "Msg " + PHRASES[idx]
    else:
        msg = "Msg " + str(idx)
    led.value(1)
    display.notify(msg)
    led.value(0)
    draw_menu()

def main():
    inp = Input(9)
    inp.on_click = on_click
    inp.on_long_click = on_long
    bus.on_message = on_receive
    bus.start_scan()
    bus.presence()

    # START: draw the first menu and idle
    draw_menu()
    while True:
        time.sleep_ms(50)  # work happens in IRQ/BLE callbacks

if __name__ == "__main__":
    main()

You can change the phrases, but it is important that both devices have the same phrases

## 8) Debugging and Errors

All programmers make mistakes — and that’s how we learn! If you see an error message in Thonny: - Read the type (e.g. IndentationError, NameError, etc.). - Look at the line number. - Fix one thing at a time and try again.

You can even make your own intentional error to see what happens!

## 9) Summary of Programming Concepts

| Concept | Description | Example |
| --- | --- | --- |
| Variable | Stores a value | score = 10 |
| Loop | Repeats code | while True: |
| Branch | Makes a choice | if ... else: |
| Function | Reusable steps | def blink(): |
| Module | Adds features | import time |
| List | Holds many items | ["YES", "NO"] |
| Dictionary | Key/value pairs | data['current'] |
| Boolean | True/False logic | if pressed: |
| Comment | Note for humans | # this does something |

## 10) Final Challenges

- Counter: Count how many times the button is pressed and display the count

- Reaction Timer: Press the button as fast as you can when the LED lights up. Show the time.

- Morse Code: Write a function that flashes the LED in SOS.

- Emoji Messenger: Display emojis when sending messages to other devices

Coding is like Scouting — you learn by doing, trying, and helping each other. Keep experimenting!